Kitchen Remodeling

Quotation for:

John Doe

123 Main Street

Cheshire, CT 06410

(203) 123-4567

July 1, 2012

The following quote, QT074112 is for Remodeling your Kitchen at your home:

- Notes

- Unless otherwise specified all material and labor is supplied by the contractor

- I believe that this quote covers all the areas of work that you would like to have remodeled, but if anything is missing or not required that already is in this quote please bring it to my attention.  Specifications in this quote can be changed per your request and the cost difference for the material & labor adjusted accordingly prior to the signing of the contract.

- Any additional work that is not part of this quote can be added by the homeowner during the remodeling work and will be quoted at that time.  The additional cost will be added to the original quote cost.  A task could also be removed and will not be billed to you.

- The following is a detail quote for your remodeling project, even with the review with you at your home prior to this quote being written some unforeseen issues may arise.  An example, a separate detail overlooked, something hidden in a wall etc. and will be reviewed with you.  The additional work scope will be quoted at that time with the extra labor hours and material costs added to this quote.

- Contractor to dispose of all his debris during the remodeling work, a 12 yard dumpster will be placed in your driveway, the wheels will be on ¾” plywood

- Will cover the doorway with plastic to help keep the dust under control and tarp the floors outside of the work area during the remodeling work

- Pick up material, order deliverables, coordinate with customer and vendors as required

- Tear down

- Remove all cabinets, counter tops, sink, and dispose of

- Will place the refrigerator on a thin board and tarp in the dining room so you could continue using it during the remodeling work

- Homeowner to make arrangements to dispose of the appliances, they can not go in the dumpster, will they be taken away when the replacement are delivered?

- Top cabinet soffits to stay as is, if you would like to remove them we will cut a few small holes in the soffit to make sure that nothing is in the soffit, wiring, plumbing etc. before they are removed, if they are removed the time and material required to remove them and to patch up the ceiling could be added to this quote

- Remove the window casement & baseboard trim in the kitchen and dining area, also remove all of the door casement trim from the door ways

- Remove the vinyl floor from the kitchen and dining area, remove the flooring under it down to the main sub floor, same subfloor that the dining room hardwood floor is fastened to

- Plumbing

- The hot & cold copper supply lines and the trap location for the sink will stay as they are, will replace the shut off valves for the hot & cold water to the sink and the hot supply to the dishwasher, contractor to also replace the supply hoses & exposed trap parts for a the sink

- New sink install will be done by the solid surface counter top installer, they will need to take it with them when they come out to make the template, they will also need to take the faucet with them too, homeowner to supply the faucet and sink, the contractor to install the faucet

- Install the sink strainer, the finish should match the faucet, chrome, brushed Nickel etc., supplied by the homeowner

- Replace the heater cover

- Electrical

- All current lights, electrical switches and outlet locations to stay as they are except for the following

- Replaces the light over the sink, supplied by the homeowner

- Will replace the center room light in the current location, light supplied by the homeowner, will remove the dining area ceiling fan during the work, will put back once the ceiling is painted

- Walls & ceiling

- Skim and patch the wall sheet rock as required to remove any popped nails crack etc., sand and ready for paint

- Skim the kitchen ceiling smooth with three coats of joint compound, sand and ready for paint

- Flooring

- The new kitchen floor stops at the doorway going to the front room and the angle doorway into the hallway

- Screw the sub floor down to the floor joists

- Install ½” cement board to the base sub floor, it will be cemented and screwed down, the tile will be flat to match the existing floor and as level as the existing floor currently is

- Will work with you laying out the pattern for the floor tile to see how much tile will be required, this quote covers installing regular tile in basic pattern and the tile being 12” up to 16” square, mosaic tile, glass inlays etc. will cost more to install and can be quoted at your request, tile supplied by the homeowner, we also offer the service of meeting with you at a tile store to review different tile selections with you and to come up with a tile pattern that you like, my brother Mike does this and the charge for this service is just his hourly rate of $50/hour for the time he works with you, we work with Tile America and get a discount on all of the tile

- Grout the lines will be 1/8” wide, caulk the tile into the surrounding surfaces as required

- The grout will have a grout sealer applied

- Tile transitions to the adjoining room will be at different heights based on the elevations  of the existing floors, oak transition pieces will be made and installed, will apply three clear coats of polyurethane to the boards but the boards will be brighter than the existing hardwood floors, also depending on how level all of the floors are the tile may be at a different height end to end as you enter an adjoining room

- Cabinet install

  - The cabinets and counter tops are supplied by the homeowner, will work with you and the supplier reviewing the layout

  - Counter tops are installed by the manufacture after I install the cabinets, the counter top manufacture will come out to make templates for each counter top, it takes about two weeks after the template is made for it to be manufactured and installed, during that time I will temporary install a small piece of your old counter top from the sink area with your old sink so you will have water in the kitchen, we will work together to set up a measure date with the manufacture once we have a target bottom cabinet install date determined, they will need the faucet and sink to review to layout the holes required to be cut

  - Homeowner to make room in the garage to store the cabinets when they come in before the job starts, I will open up the boxes and inspect for any damage before the work starts, the sooner they are inspected the better so if any cabinet is damaged it can be replace prior to the start date

  - Will install all of the cabinets, base and uppers; install all trim pieces supplied etc.

  - Install the new stove, microwave, refrigerator in the existing alcove (no icemaker line) & dishwasher, all supplied by the homeowner

  - The microwave will be installed with the unit venting into the kitchen thru the front louvers, could review the inside the soffit to see if the vent could be run to the outside of the house, if so this task will be quoted for your review

- Trim & accessories

  - All trim to be standard colonial 2 ½” casement trim for the windows and door ways & 3 ½” for the baseboard trim

  - The kitchen window will stay as is

- Back Splash tile

  - Install back splash tile from the top of the counter top to the underside of the upper cabinets, the tile will go up each side of the window trim about mid way and up to the underside of the microwave, will pull out the stove so the tile could go behind the stove and one row below the horizontal counter top line by the stove, this is for the stove/sink area

  - Will work with you laying out the pattern for the backsplash tile to see how much tile will be required, this quote covers installing the tile in basic patterns and the tile being at least 4” square, mosaic tile, glass inlays etc. will cost more to install and can be quoted at your request, tile supplied by the homeowner, we also offer the service of meeting with you at a tile store to review different tile selections with you and to come up with a tile pattern that you like, my brother Mike does this and the charge for this service is just his hourly rate of $50/hour for the time he works with you, we work with Tile America and get a discount on all of the tile

  - Grout the 1/8” grout lines, caulk the tile into the surrounding surfaces as required

  - Will put plastic electrical spacers and install longer screws in the switches and outlets so they sit on top of the new tile surface

  - The grout will have a grout sealer applied

  - Will review with you to determine where the end of the vertical tile should stop in relationship to the end of the counter top, also review how the ends should be finished, edges cover with wood or a deco piece of tile to dress it up

- Painting

  - Caulk all trim into the sheet rock & mating trim, fill all nail holes & sand

  - Kitchen and dining area painting - apply one coat of primer to all the walls, ceiling & trim, top coat the trim with a semi gloss paint, paint both sides of the three new doors, sand the three stained doorways to remove the smooth finish and to rough up the stain, apply two coats of a flat white to the ceiling, apply two coats of an eggshell paint to the walls, all paint by the Benjamin Moore Company, excluding the Aura paint

- Customer supplied items

- The following is a list of the items that throughout this quote are called out as “supplied by the homeowner”

| Item # | Description | Estimated costs (not included in this quote) |
| --- | --- | --- |
| 1 | Sink, faucet & strainer, home center | $535 |
| 2 | Kitchen center room lights and light over the sink, home center | $195 |
| 3 | Floor tile, Tile America | $1,140 |
| 4 | All appliances, existing or new | Open item! |
| 5 | Granite counter tops, Viking Kitchens | $3,100 |
| 6 | All cabinets, Viking Kitchens | $4,830 |
| 7 | Back splash tile, Tile America | $350 |
|  |  | $10,150 |

ACCEPTANCE OF PROPOSAL

We hereby propose to furnish labor and materials – complete in accordance with the above specifications, for the sum of $14,995 - $18,355, this excludes the “Homeowner supplied material” estimated costs on the previous page, if that estimated cost, ($10,150) was applied to this quote the estimated range would become $25,145 - $28,505, mid range of $26,825

Our goal is to have the cost come in at the mid range for our labor, $12,400 and the contractor supplied material, $4,275 but we give a range so the cost could be lower than the mid range and the cost could also come in at the upper end of the estimate.  Our upper end for the contractor labor and material is a not to exceed cost as long as the work scope is not modified by the homeowner from the original quote.  The customer supplied material is an estimate for the items you supply, this cost is under your control based on what you choose.

A$200 deposit is due upon signing the contract.  Material that is required to be ordered before the projects starts needs to be paid for in advance, homeowner can pay directly to the vendors.  Material will be billed by the receipts submitted at the end of each week of work; there is no mark up on the material.  The estimated contractor supplied material range is $3,920 - $4,630.  The estimated labor range is $11,075 - $13,725, employee labor rate are $45 - $55 per hour.  The weekly total hours will be submitted at the end of each week with a description of the work that was completed along with the material receipts, payment is due then or by the following Monday.  If the job is finished in a given work week the payment is due upon the job completion.

All material is guaranteed to be as specified.  All work to be completed in a workmanlike manner according to standard practices.  Any alteration or deviation from above specifications involving a cost change will be excluded only upon written orders, and will become a cost credit or deletion to the above quote.  Adding additional work to the above specification may not be completed in order with the outlined work and is subject to be scheduled at the contractor and homeowner’s discretion.  All agreements are contingent upon weather, accidents, or delays beyond our control.

### Authorized Signature,       Stuart J. Glatt, 7/1/12

This quote, #QT074112 is subject to acceptance, signed by the homeowner and returned to the contractor within seven (7) calendar days from the above date and is void thereafter at the option of the undersigned.  Work start to be scheduled.

The above prices, specifications and conditions are hereby accepted.  You are authorized to do the work as specified. Payment will be made as outlined above.

ACCEPTED:

Signature/date